Units in the Simulator

Genesis elements can store numerical values, but these values are not required

to be expressed in any particular system of units.  However, various routines

that make use of these values naturally assume that the values are consistent

with each other in some framework of scientific units.

The issue of units is a tricky one: in any case, you will want to be sure your

use of numerical values is consistent with a single set of units, both within

and between components of the model.

The remainder of this section considers several self-consistent approaches to

physiological units that work with the neural elements in Genesis.

Compartmental Units

The units of the basic compartmental equation are as follows:

voltage = (time/capacitance)*(voltage/Rm + voltage/Ra + current)

Common physiological units (per unit area) are:

---------------------------------------------

Symbol    Meaning                 Units

---------------------------------------------

Im        Membrane Current        (uA/cm^2)

Vm        Membrane Voltage        (mV)

Em        Reversal Potential      (mV)

Rm        Membrane Resistance     (Kohm-cm^2)

Cm        Membrane Capacitance    (uF/cm^2)

inject    Injected Current        (uA/cm^2)

---------------------------------------------

time = msec

(Ra doesn't apply when considering unit area membranes (Ra = Kohm-cm).)

The equivalent units with dimensional information factored in are as follows:

----------------------------------------

Symbol    Meaning                 Units

----------------------------------------

Im        Membrane Current        (uA)

Vm        Membrane Voltage        (mV)

Em        Reversal Potential      (mV)

Rm        Membrane Resistance     (Kohm)

Ra        Axial Resistance        (Kohm)

Cm        Membrane Capacitance    (uF)

inject    Injected Current        (uA)

----------------------------------------

time = msec

Note that time units are also affected by the choice of membrane units.  Also

note that the compartment has no intrinsic assumption of area.  A consistency

check of these units based on the compartmental equation earlier shows that

these two systems check:

mV = (msec/uF)*(mV/Kohm + uA)

e-3 = (e-3/e-6)*(e-3/e+3 + e-6)

e-3 = (e+3)*(e-6)

e-3 = e-3

Other units which are consistent (and are used in one of the sample

simulations) are as follows:

----------------------------------------

Symbol    Meaning                 Units

----------------------------------------

Im        Membrane Current        (A)

Vm        Membrane Voltage        (V)

Em        Reversal Potential      (V)

Rm        Membrane Resistance     (ohms)

Cm        Membrane Capacitance    (F)

Ra        Axial Resistance        (ohms)

inject    Injected Current        (A)

----------------------------------------

time = seconds

Again, whatever unit system you choose, be sure you are consistent!